# SAFEVISION-FYP-RD40

# Computer Vision Based Detection of

# Zero to Low Visibility Conditions

Product Specification Sheet

| Institution NUCES-FAST, Islamabad STAIR LAB | Supervisor Dr. Muhammad Tariq | Team Members Abdullah Ali (i22-2153) Ali Shahid (i22-2199) Zain Khalid (i22-2161) |
| --- | --- | --- |

# PRODUCT OVERVIEW

The SAFEVISION-FYP-RD40 system is a low-cost, AI-powered, multi-sensor edge platform designed to detect obstacles and road hazards in real-time under zero to low visibility conditions caused by dense fog and smog. The system fuses data from RGB cameras, thermal sensors, and mmWave radar to ensure uninterrupted detection performance even when individual sensors fail.

KEY FEATURES

- Triple-modality sensor fusion: RGB camera + thermal (MLX90640) + 24 GHz mmWave radar (RD03-V2)

- Adaptive sensor switching: automatically selects the most reliable data source in real time

- YOLO-based object detection and classification for vehicles, pedestrians, and obstacles

- Edge inference on Raspberry Pi 5 no cloud dependency

- Real-time driver alerts via buzzer module with instant notification on hazard detection

- YOLO architecture: camera-radar fusion backbone for enhanced detection in whiteout conditions

All processing runs on-device using and Raspberry Pi 5 with YOLO-based object detection delivering edge-level inference with no cloud dependency, sub second latency, and immediate buzzer alerts for drivers.

# Technical SPECIFICATIONS

Processing & Software

| Parameter | Specification |
| --- | --- |
| Processor | Raspberry Pi 5 |
| Object Detection Model | YOLO26 |
| Software Stack | Python, OpenCV, Deep Learning framework |
| Inference Mode | Edge (local) ; no cloud dependency |
| Detection Latency | Real-time, sub-second edge processing |

SENSOR SPECIFICATIONS

RGB Camera

| Parameter | Specification |
| --- | --- |
| Type | RGB visible-light camera |
| Function | Primary detection under normal/partial visibility |
| Processing | YOLO pipeline on Raspberry via OpenCV |
| Failover | Automatically deprioritized in dense fog/smog |

Thermal Sensor

| Parameter | Specification |
| --- | --- |
| Model | MLX90640 |
| Type | Far-infrared thermopile array |
| Function | Heat signature detection in fog / low light |
| Interface | I2C |
| Role in Fusion | Takes over from camera in poor optical visibility |

mmWave Radar

| Parameter | Specification |
| --- | --- |
| Model | RD03-V2 |
| Frequency Band | 24 GHz K-band |
| Function | Obstacle ranging and detection regardless of visibility |
| Key Advantage | Immune to fog, dust, and darkness |
| Role in Fusion | Provides bounding-box data to CR-YOLO network |

# SYSTEM ARCHITECTURE

Sensor Fusion Pipeline

The three sensor modalities feed into a Sensor Fusion module with adaptive switching logic. Based on real-time signal quality assessment, the module selects and weights inputs before forwarding to the AI processing stage.

| Stage | Description |
| --- | --- |
| Stage 1 — Sensing | RGB Camera + Thermal Sensor + Radar Sensor array capture environmental data simultaneously |
| Stage 2 — Fusion & Switching | Adaptive switching module evaluates each source and routes the most reliable feed(s) to the AI pipeline |
| Stage 3 — AI Processing | Raspberry Pi YOLO / CR-YOLO for obstacle detection, classification, and bounding box generation |
| Stage 4 — Driver Alert | Instant buzzer notification triggered on hazard detection; future: mobile app broadcast |

CR-YOLO Network Architecture

The CR-YOLO (Camera-Radar YOLO) network processes both radar point cloud data and RGB camera frames through dual backbone networks. Feature maps are fused at the neck layer and passed to detection heads for simultaneous object classification and bounding box prediction.

| Component | Description |
| --- | --- |
| Radar Backbone | Processes radar signal projection into feature space |
| Camera Backbone | Processes RGB image features |
| Feature Fusion (Neck) | Merges radar and camera feature tensors |
| Detection Head | Outputs class labels + bounding boxes |
| Output | Object detection with classification and localization |

# PERFORMANCE SPECIFICATIONS

| Metric | Value |
| --- | --- |
| Detection Accuracy (AI + Radar) | 93% |
| Detection Accuracy (Camera Only) | 67% |
| Performance Gain over Baseline | +26 percentage points |
| Processing Mode | Real-time edge inference |
| Alert Latency | Immediate (buzzer on detection) |
| Visibility Conditions Supported | Clear, foggy, smoggy, nighttime |
| Sensor Failover | Automatic, real-time switching |

# HARDWARE & DEPLOYMENT

Bill of Materials

| Component | Model / Specification |
| --- | --- |
| Edge Controller | Raspberry Pi 5 |
| RGB Camera | Standard USB/CSI camera module |
| Thermal Sensor | MLX90640 (far-infrared array) |
| Radar Module | RD03-V2, 24 GHz K-band mmWave |
| Driver Alert | Buzzer module (edge-triggered) |
| Development Environment | Python, OpenCV, Visual Studio Code |

Physical Deployment

| Attribute | Specification |
| --- | --- |
| Deployment Platform | Vehicle-mounted or roadside unit |
| Power Source | 12V vehicle power or portable battery pack |
| Connectivity | Standalone edge device; no internet required |
| IP Rating | Hardware enclosure recommended for outdoor use |
| Scalability | Modular design; supports additional sensor integration |

# APPLICATIONS

- Highway driver assistance during dense fog and smog events

- Smart vehicle and ADAS (Advanced Driver Assistance System) integration

- Autonomous driving platform perception layer

- Smart city road infrastructure safety monitoring

- Emergency vehicle navigation in zero-visibility conditions

- Research testbed for multi-sensor fusion algorithms

# FUTURE DEVELOPMENT ROADMAP

| Feature | Description |
| --- | --- |
| Long-Range Sensor Integration | Extended radar range for highway early-warning |
| Mobile App Alert Broadcasting | Real-time hazard push notifications to driver smartphone |
| 3D Scene Visualization | Point-cloud and depth map overlay for situational awareness |
| Cloud Analytics Dashboard | Fleet-level fog/smog event logging and heatmaps |
| Enhanced YOLO Model | Fine-tuned on local fog/smog datasets for higher accuracy |

# SDG ALIGNMENT

| SDG 3 — Good Health & Well-Being | Reduces fog-related road fatalities and injuries through timely hazard detection |
| --- | --- |
| SDG 8 — Decent Work & Economic Growth | Lowers economic costs of road accidents; enables scalable, affordable safety tech |
| SDG 9 — Industry, Innovation & Infrastructure | Advances smart transport infrastructure with AI-driven sensor fusion |
| SDG 11 — Sustainable Cities & Communities | Supports safer urban mobility and smarter city road safety systems |

PRODUCT INFORMATION

# Model: SAFEVISION-FYP-RD40

Version: 1.0

Date: May 2026

Document Type: Product Specification Sheet

IMPORTANT NOTICE

This system is developed for research, academic, and demonstration purposes as part of a Final Year Project at NUCES-FAST. Specifications are subject to change. Performance may vary based on environmental conditions, sensor calibration, and hardware configuration. Users are responsible for safe deployment and compliance with local regulations.