SAFEVISION-FYP-RD40

Computer Vision Based Detection of Zero to Low Visibility Conditions

USER MANUAL

⚠ PLEASE READ THIS MANUAL CAREFULLY BEFORE USE ⚠ • This manual contains important safety information and operating instructions. • Keep this manual for future reference. Failure to follow instructions may result in equipment damage, incorrect readings, or unsafe operation.

Thank you for choosing Safe Vision!

This manual will guide you through setup, operation, and maintenance of your intelligent low-visibility detection platform.

Team Members 1. Zain Khalid R. No: 22I-2161 2. Abdullah Ali R. No: 22I-2153 3. Ali Shahid R. No: 22I-2199

TABLE OF CONTENTS

1.  Safety Information

2.  Introduction

3.  Package Contents

4.  System Overview

5.  Initial Setup

6.  Software Installation

7.  Operating Instructions

8.  Pre-Operation Checklist

9.  Running Detection Missions

10.  Telemetry and Monitoring

11.  Troubleshooting

12.  Maintenance and Care

13.  Power Safety

14.  Technical Specifications

15.  Warranty and Support

1.  SAFETY INFORMATION

⚠ WARNING ⚠ • Always keep visual line of sight with the platform during operation. • Keep all spectators at least 2 meters away from the robot. • Do not operate near water, on unstable surfaces, or in heavy rain. • Always use a regulated 25W power adapter for Raspberry Pi 5. • Disconnect power before changing breadboard or GPIO wiring.

General Safety Precautions

- Read and understand all instructions before operating.

- Do not modify hardware or code without proper validation.

- Use only tested sensor wiring for RD-03, MLX90640, and LCD modules.

- Keep the platform away from children and pets during testing.

- Do not run if any module appears damaged or overheated.

2.  INTRODUCTION

Safe Vision is a final year project designed for object and threat detection in zero to low visibility environments. The system combines USB camera vision, AI Thinker RD-03 radar, and MLX90640 thermal sensing on Raspberry Pi 5.

Key Capabilities

- Custom YOLOv26 model trained on a low-visibility custom dataset.

- Three target detection classes: car, person, and bike.

- Dynamic weighted average sensor fusion (camera + radar + thermal).

- Real-time threat estimation and alerting via buzzer, LCD, and display UI.

3.  PACKAGE CONTENTS

Please verify that your package includes all of the following items:

| Qty | Item | ✓ |
| --- | --- | --- |
| 1 | Raspberry Pi 5 board |  |
| 1 | USB HD camera |  |
| 1 | AI Thinker RD-03 radar module |  |
| 1 | MLX90640 thermal sensor |  |
| 1 | 16×2 LCD module |  |
| 1 | Buzzer module |  |
| 1 | 25W power adapter |  |
| 1 | Breadboard and jumper wiring set |  |
| 1 | MicroSD card with software/code |  |
| 1 | User Manual (this document) |  |

Note: If any items are missing or damaged, please contact the project team immediately.

4.  SYSTEM OVERVIEW

Component Identification

Familiarize yourself with the main components of Safe Vision:

- Raspberry Pi 5 — main processor and control unit.

- USB HD Camera — provides frames for YOLOv26 inference.

- RD-03 Radar — distance sensing and obstacle proximity input.

- MLX90640 Thermal Sensor — thermal threat profile and body heat detection.

- 16×2 LCD — local text display for distance and temperature state.

- Buzzer — acoustic alert for critical threshold events.

Status Indicators

- LCD startup text confirms the system initialisation sequence.

- Live UI shows dominant sensor and fused threat bar (0–100%).

- Buzzer activates on radar, thermal, or fused-threat alert conditions.

5.  INITIAL SETUP

Step 1 — Hardware Inspection

- Verify all sensor modules are mounted and secure.

- Confirm breadboard wiring continuity.

- Check camera USB and radar UART connections.

- Confirm LCD and buzzer GPIO wiring.

Step 2 — Power Preparation

- Connect only a stable 25W power adapter.

- Ensure Raspberry Pi 5 boots without undervoltage warning.

- Do not power from unstable or low-current sources.

Step 3 — First Power-On Test

- Power ON and wait for LCD initialization message.

- Confirm camera feed opens correctly.

- Confirm radar serial data is being received.

- Confirm thermal sensor is detected on I2C.

6.  SOFTWARE INSTALLATION

Required Software

- Python 3.10 or later

- OpenCV (cv2), NumPy, pyserial

- ultralytics — YOLO model runtime

- lgpio — buzzer and LCD GPIO pin control

- adafruit-circuitpython-mlx90640, board, busio — thermal sensor

Python Library Installation

pip install opencv-python pip install ultralytics pip install numpy pip install pyserial pip install lgpio pip install adafruit-circuitpython-mlx90640

Installation Steps

- Install Raspberry Pi OS and update all packages.

- Install all Python dependencies listed above.

- Copy YOLO weights file best1.pt into the project directory.

- Connect camera, radar, thermal sensor, LCD, and buzzer.

- Run the main fusion script.

python3 main.py

7.  OPERATING INSTRUCTIONS

Starting the System

- Connect power adapter and boot Raspberry Pi 5.

- Launch the main script: python3 main.py

- Verify the on-screen fusion display appears.

- Confirm live updates for radar distance and thermal temperature.

8.  PRE-OPERATION CHECKLIST

Complete this checklist before every detection session:

| Checkpoint | ✓ |
| --- | --- |
| Power adapter connected and stable |  |
| Camera feed active |  |
| Radar serial values updating |  |
| Thermal sensor available on I2C |  |
| YOLO model loaded (best1.pt) |  |
| LCD output readable |  |
| Buzzer test functional |  |
| Operating area clear and safe |  |

9.  RUNNING DETECTION MISSIONS

Standard Mission Flow

- Start the script and allow all sensors to initialize.

- Position platform in the target low-visibility scene.

- Observe YOLO detections for car, person, and bike classes.

- Monitor the fused threat percentage and dominant sensor weighting.

- Respond to radar, thermal, or fused-threat alerts immediately.

Alert Thresholds

| Sensor | Alert Condition | Action |
| --- | --- | --- |
| Radar | Distance < 300 cm | Buzzer ON + LCD warning |
| Thermal | Temperature > 36.0 °C | Buzzer ON + LCD alert |
| Fused | Threat level ≥ 45% | Buzzer ON + UI alert |

10.  TELEMETRY AND MONITORING

Displayed Runtime Data

- Fused threat bar (0–100%)

- Dynamic sensor weights: camera, radar, thermal

- Dominant sensor state

- Thermal status and maximum temperature reading

- Radar distance and risk state

Fusion Details

- Camera confidence: derived from brightness, contrast, and YOLO detection confidence.

- Radar confidence: derived from number of valid data points and nearest distance.

- Thermal confidence: derived from valid temperature range behavior.

- Weights are clipped, normalized, and smoothed over time for stability.

11.  TROUBLESHOOTING

Common Issues and Solutions

| Problem | Possible Cause | Solution |
| --- | --- | --- |
| No camera feed | USB issue or wrong camera index | Reconnect camera; verify cv2.VideoCapture(0). |
| Radar not detected | Wrong UART wiring or port | Check /dev/ttyAMA0, baud 115200, TX/RX lines. |
| Thermal unavailable | I2C or library issue | Check SDA/SCL wiring; reinstall MLX90640 libs. |
| No object detections | Model file not found | Confirm best1.pt is in the project directory. |
| Buzzer always on | Threshold repeatedly exceeded | Inspect radar noise and thermal threshold logic. |
| LCD not updating | GPIO pin mismatch | Verify RS/E/D4–D7 pin mapping in code. |

12.  MAINTENANCE AND CARE

After Each Use

- Power down safely and disconnect the adapter.

- Check breadboard and jumper wire integrity.

- Clean camera lens and sensor surfaces.

- Store in a dry environment away from dust and moisture.

Weekly

- Inspect GPIO and serial connectors for looseness.

- Test buzzer, LCD, radar, and thermal modules individually.

- Backup source code and YOLO weights.

Storage Recommendations

- Store in a dry location between 15 °C and 35 °C.

- Disconnect power adapter after every use.

- Protect sensors from direct sunlight and heavy dust.

13.  POWER SAFETY

⚠ CRITICAL POWER WARNINGS ⚠ • NEVER use unregulated or low-current adapters. • NEVER short power rails on the breadboard. • NEVER change power wiring while the system is ON. • Immediately disconnect power if overheating is detected.

Safe Power Practices

- Always use the supplied 25W regulated power adapter.

- Ensure the Raspberry Pi 5 boot log shows no undervoltage warnings.

- Shut down the OS properly before disconnecting power.

14.  TECHNICAL SPECIFICATIONS

Quick Reference Guide

| Parameter | Value |
| --- | --- |
| Model | SAFEVISION-FYP-RD40 |
| Core Controller | Raspberry Pi 5 |
| Vision Sensor | USB HD Camera |
| Radar Sensor | AI Thinker RD-03 |
| Thermal Sensor | MLX90640 |
| Detection Model | YOLO26 — custom trained |
| Detection Classes | Car, Person, Bike |
| Sensor Fusion Method | Dynamic Weighted Average |
| Alert Outputs | Buzzer + 16×2 LCD + OpenCV UI |
| Power Source | 25W Regulated Adapter |
| Programming Language | Python 3.10+ |
| Communication | UART, I2C, USB |
| Operating Environment | Low Visibility / Fog / Dark |

15.  WARRANTY AND SUPPORT

Academic Prototype Notice

This product is an academic prototype provided for educational and research demonstration purposes. It is not intended for commercial deployment without further validation.

Support Guidelines

- The project team should maintain the latest tested version of code and model weights.

- Keep the wiring diagram and pin map with the system at all times.

- Record any changes to thresholds and fusion parameters for reproducibility.

For technical questions, contact the project team directly. Please include system logs and wiring configuration details when reporting issues.

Thank you for using Safe Vision!

Computer Vision Based Detection of Zero to Low Visibility

Final Year Project — Embedded AI and Sensor Fusion System